# FIAMM DUEL ELECTRIC HORNS

The kit comes with both a high & low tone horn and a relay.

## Before you start:-

Items you will need:-

- an inline fuse holder ( with fuse 5 amp)

- Electrical cable multi core (Flexible) rated 8 amp

- Insulating tape

- 4 x spade connectors, female type (Blue 15 amp ones from Hal fords)

Tools needed

- Crimping tool for spade connectors (you can use a pair of pliers for this if care is taken).

- 10mm spanner.

- Cross head screwdriver.

- Allen keys.

- Knife

- Cable ties.

- Hot glue gun (optional).

## Time:- This job should take between 30 min-1hour to complete.

## Fitting:-

- remove the trim panel from the inside of the mirror base offside ( right as you sit on bike)

- remove the mirror

- disconnect the indicator

- undo the Allen bolts retaining the offside fairing, panel and remove

- undo the two retaining bolts and raise the tank

- disconnect the wires running to the horn (in front of radiator)

- undo the single retaining 10mm bolt and remove the horn and bracket

- Cut the lugs joining the two horn wire connectors together

- Feed the wires back through the frame, into the space behind the headstock.

- pull the wires up in front of the air box

- undo the 10mm nut off the back of both horns, remove the mounting bracket

- Bend mounting bracket  from Fig.1 into Fig.2, by making two right angled bends (this enables them to clear the forks on full lock)

- undo the offside 10mm radiator top mounting bolt

- Slide the horn bracket between the frame lug and the radiator mount, reinstall the bolt.

- repeat for other side ( this can be done without removing the fairing panel)

- mount the horns onto the fitted brackets ( trumpets down)

- Utilizing the 5 amp inline fuse, install a direct feed from the live terminal of the battery to the front of the air box, terminating with a female spade connector (follow the loom around the nearside of the frame.).

- cut two lengths of the electrical cable 10 inches long, terminate one end of each with a spade connector(female)

- terminate the other end of both cables in the same spade connector( making a length of wire with a spade connector at each end and one in the middle)

- connect each end of this cable to the single spade connector on the horn

- connect the middle spade to the relay

- connect the two original wires to the relay, as shown

- Connect the direct feed from the battery to the relay.

- Test.

- insulate all connections with tape (or heat shrink)

- Lift the breather pipe from the front of the air box and fix the relay (using a hot glue gun) to the front of the air box.

- Cable tie the direct feed wire to the loom.

- lower tank to cover relay, and replace bolts

- refit fairing panel , mirror & trim panel

### NOW GO AND MAKE SOME NOISE!!!!!!!